Battery Charging Circuit

Designed by reading datasheet of BQ25100

- System Load Determination

{1.22mA * 3.3V*2(dual amp opa2392) + 7.3mA*1.8V(nRF MCU) + 70uW(ADC)}/90%(efficiency)*3.7V(nom. Lipo voltage) = 6.4mA

- K_set : 135Aohm , I_out = 60mA => Resistor on I_set : 2.25kohm => 2.2kohm

- Actual fast charge current: 61.36mA, 배터리에 들어가는 전류: 54.98mA

Resistor on TS: tie with GND using 10kohm resistor

Resistor on PRETERM:

Termination current: 6.1(10% of total fast charge current) + 6.4 = 12.5mA

Resistor on PRETERM: 600ohm/% * (100* 12.5mA/61.36mA) = 12223ohm => 12.1kohm

When system is ON, Actual termination current: 15.03mA, 배터리에 들어가는 전류: 5.03mA 때 종료

Resistor on PRETERM(OFF PATH): 600ohm/% * (100* 6.1mA/61.36mA) = 6kohm total with 12.1kohm parallel

- 12.0kohm

When system Is OFF, Actual termination current: 5.89mA